ANNEX  II.     PART - ARO

AUTHORITY  REQUIREMENTS  FOR  AIR  OPERATIONS

In  Annex  II  to  CR - EU  No  965 / 2012,  the  following  points :

ARO. OPS. 230   and  ARO. OPS. 235   are  added.

INTENTIONALLY            LEFT            BLANK

ANNEX  II.     PART - ARO

AUTHORITY    REQUIREMENTS    for    AIR    OPERATIONS

CONTENT

Content    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .     3

Record  of  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    5

Record  of  Temporary  Revision    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    7

List  of  Effective  Pages ( LEP )    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    9

ARO. GEN. 005   Scope    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   11

SUBPART   GEN.    General  Requirements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  13

Section  I.      General    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    13

ARO. GEN. 115   Oversight  Documentation   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  13

ARO. GEN. 120   Means  of  Compliance   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  13

ARO. GEN. 125   Information  to  the  Agency   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   13

ARO. GEN. 135   Immediate  Reaction  to  a  Safety  Problem   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  14

Section  II.    Management   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   15

ARO. GEN. 200   Management  System   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  . 15

ARO. GEN. 205   Allocation  of  Tasks  to  Qualified  Entities   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  15

ARO. GEN. 210   Changes  in  the  Management  System   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    16

ARO. GEN. 220   Record - Keeping   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  16

ARO. OPS. 230    Determination   of   Disruptive   Schedules    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   17

ARO. OPS. 235    Approval  of  Individual  Flight  Time Specification  Schemes   .  .  .  .  .  .  .   17

Section  III    Oversight,  Certification  and  Enforcement    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    19

ARO. GEN. 300   Oversight   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   19

ARO. GEN. 305   Oversight  Programme   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   19

ARO. GEN. 310   Initial Certification Procedure - Organisations  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   20

ARO. GEN. 330   Changes - Organisations   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   20

ARO. GEN. 345   Declaration  -  Organization’s   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  . 21

ARO. GEN. 350   Findings  and  Corrective  Actions  -  Organization’s   .  .  .  .  .  .  .  .  .  .  .  .  . 21

ARO. GEN. 355   Findings  and  Enforcement  Measures  -  Persons   .  .  .  .  .  .  .  .  .  .  .  .  .  .   22 ARO. GEN. 360   Findings and Enforcement Measures  - all Operators  .  .  .  .  .  .  .  .  .  .  .  .  .   22

SUBPART   OPS.      Air  Operation’s    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    23

Section  I.      Certification  of  Commercial  Air  Transport  Operators   .  .  .  .  .  .  .  .  .  .    23

ARO. OPS. 100   Issue  of  the  Air  Operator  Certificate   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  23

ARO. OPS. 105   Code - Share  Arrangements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    23

ARO. OPS. 110   Lease  Agreements   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    23

SECTION  I a   Authorization of High Risk Commercial  Specialized  Operations  .  .  .  .  25

ARO. OPS. 150  Authorization of  High Risk Commercial Specialized Operations   .  .  .  .  .  .   25

ARO. OPS. 155  Lease Agreements  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   25

Section  II.    Approvals    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    27

ARO. OPS. 200   Specific  Approval  Procedure   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  . 27

ARO. OPS. 205   Minimum  Equipment  List  Approval  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  27

ARO. OPS. 210   Determination of  Distance or Local Area   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    27

ARO. OPS. 215   Approval  of  Helicopter  Operations  Over  a  Hostile  Environment  Located

Outside   a  Congested  Area   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   27

ARO. OPS. 220   Approval  of  Helicopter  Operations  to  or  from  a  Public  Interest  Site   .  .  28

ARO. OPS. 225   Approval  of  Operations  to  an  Isolated  Aerodrome   .  .  .  .  .  .  .  .  .  .  .  .    28

SECTION  III.   Oversight  of  Operations  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   29

ARO. OPS. 300   Introductory  Flights   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   29

SUBPART   RAMP.   Ramp  Inspections  of  Aircraft  of  Operators  under  the

Regulatory  Oversight  of  another  State   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  31

ARO. RAMP. 005   Scope   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    31

ARO. RAMP. 100   General    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   31

ARO. RAMP. 105   Prioritisation  Criteria   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   31

ARO. RAMP. 110   Collection  of  Information   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    32

ARO. RAMP. 115   Qualification  of  Ramp  Inspectors   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   32

ARO. RAMP. 120   Approval  of  Training  Organisations   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  32

ARO. RAMP. 125   Conduct  of  Ramp  Inspections   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  33

ARO. RAMP. 130   Categorization  of  Findings   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  33

ARO. RAMP. 135   Follow - up  Actions  on  Findings   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  . 33

ARO. RAMP. 140   Grounding  of  Aircraft   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    34

ARO. RAMP. 145   Reporting   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    34

ARO. RAMP. 150   Agency  ( EASA )  Coordination  Tasks   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    35

ARO. RAMP. 155   Annual   Report   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    35

ARO. RAMP. 160    Information to the Public and  Protection of Information   .  .  .  .  .  .  .  .  .  .  35

GUIDANCE  MATERIAL  &  ACCEPTABLE  MEANS  of  COMPLIANCE   .  .  .  .  .  .  .    37

Appendix  I.    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    107

Appendix  II.   .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   109

Appendix  III.    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    111

Appendix  IV.    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    113

Appendix  V.    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .   117

Appendix  VI.    .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .  .    119

RECORD     OF    REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

RECORD     OF    REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

RECORD   of   TEMPORARY   REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

RECORD   of   TEMPORARY   REVISION

| Rev No | Date of issue | Date of entered | Entered by [ Name & Position ] |
| --- | --- | --- | --- |

LIST    OF    EFFECTIVE    PAGES

| I T E M | FROM PAGE | TO PAGE |  |  | EFFECTIVE DATE |
| --- | --- | --- | --- | --- | --- |
| CONTENT | 3 | 4 |  |  | 01 - November - 2014 |
| RECORDS of REVISION | 5 | 6 |  |  | 01 - November - 2014 |
| RECORDS of TEMPORARY REVISION | 7 | 8 |  |  | 01 - November - 2014 |
| LIST of EFFECTIVE PAGE | 9 | 10 |  |  | 01 - November - 2014 |
| ARO. GEN. 005 Scope | 11 | 12 |  |  | 01 - November - 2014 |
| SUBPART GEN. GENERAL REQUIREMENTS | 13 | 22 |  |  | 01 - November - 2014 |
| Section I. General | 13 | 14 |  |  | 01 - November - 2014 |
| ARO. GEN. 115 Oversight Documentation | 13 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 120 Means of Compliance | 13 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 125 Information to the Agency | 13 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 135 Immediate Reaction to a Safety Problem | 14 |  |  |  | 01 - November - 2014 |
| Section II. Management | 15 | 18 |  |  | 01 - November - 2014 |
| ARO. GEN. 200 Management System | 15 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 205 Allocation of Tasks to Qualified Entities | 15 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 210 Changes in the Management System | 16 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 220 Record - Keeping | 16 |  |  |  | 01 - November - 2014 |
| ARO. OPS. 230 Determination of Disruptive Schedules | 16 |  |  |  | 01 - November - 2014 |
| ARO. OPS. 235 Approval of Individual Flight Time Specification Schemes | 17 |  |  |  | 01 - November - 2014 |
| Section III. Oversight, Certification and Enforcement | 19 | 22 |  |  | 01 - November - 2014 |
| ARO. GEN. 300 Oversight | 19 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 305 Oversight Programme | 19 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 310 Initial Certification Procedure - Organizations | 20 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 330 Changes - Organizations | 20 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 345 Declaration - Organizations | 21 |  |  |  | 01 - November - 2014 |
| ARO.GEN.350 Findings and Corrective Actions - - Organizations | 21 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 355 Findings and Enforcement Measures - - Persons | 21 |  |  |  | 01 - November - 2014 |
| ARO. GEN. 360 Findings and Enforcement Measures - all Operators | 22 |  |  |  | 01 - November - 2014 |
| SUBPART OPS. AIR OPERATION’s | 23 |  |  | 30 | 01 - November - 2014 |
| Section I. Certification of Commercial Air | 23 |  |  | 24 | 01 - November - 2014 |
| ARO. OPS. 100 Issue of the Air Operator Certificate | 23 |  |  |  | 01 - November - 2014 |
| ARO. OPS. 105 Code - Share Arrangements | 24 |  |  |  | 01 - November - 2014 |
| ARO. OPS. 110 Lease Agreements | 24 |  |  |  | 01 - November - 2014 |
| Section I a. Authorization of High Risk Commercial Specialized Operations Transport Operators | 25 |  | 26 |  | 01 - November - 2014 |
| ARO. OPS. 150 Authorization of High Risk Commercial Specialized Operations | 25 |  |  |  | 01 - November - 2014 |
| ARO. OPS. 155 Lease Agreements | 25 |  |  |  | 01 - November - 2014 |
| ARO. OPS. 210 Determination of Distance or Local Area | 25 |  |  |  | 01 - November - 2014 |

LIST    OF    EFFECTIVE    PAGES

| I T E M | FROM PAGE |  | TO PAGE | EFFECTIVE DATE |
| --- | --- | --- | --- | --- |
| Section II. Approvals | 27 |  | 28 | 01 - November - 2014 |
| ARO. OPS. 200 Specific Approval Procedure | 27 |  |  | 01 - November - 2014 |
| ARO. OPS. 205 Minimum Equipment List Approval | 27 |  |  | 01 - November - 2014 |
| ARO. OPS. 210 Determination of Local Area | 27 |  |  | 01 - November - 2014 |
| ARO. OPS. 215 Approval of Helicopter Operations Over a Hostile Environment Located Outside a Congested Area | 27 |  |  | 01 - November - 2014 |
| ARO. OPS. 220 Approval of Helicopter Operations to or from a Public Interest Site | 28 |  |  | 01 - November - 2014 |
| ARO. OPS. 225 Approval of Operations to an Isolated Aerodrome | 28 |  |  | 01 - November - 2014 |
| Section III. Oversight of Operations | 29 | 30 |  | 01 - November - 2014 |
| ARO. OPS. 300 Introductory Flights | 29 |  |  | 01 - November - 2014 |
| SUBPART RAMP. Ramp Inspections of Aircraft of Operators under the Regulatory Oversight of another State | 31 | 36 |  | 01 - November - 2014 |
| ARO. RAMP. 005 Scope | 31 |  |  | 01 - November - 2014 |
| ARO. RAMP. 100 General | 31 |  |  | 01 - November - 2014 |
| ARO. RAMP. 105 Prioritisation Criteria | 31 |  |  | 01 - November - 2014 |
| ARO. RAMP. 110 Collection of Information | 32 |  |  | 01 - November - 2014 |
| ARO. RAMP. 115 Qualification of Ramp Inspectors | 32 |  |  | 01 - November - 2014 |
| ARO. RAMP. 120 Approval of Training Organisations | 32 |  |  | 01 - November - 2014 |
| ARO. RAMP. 125 Conduct of Ramp Inspections | 33 |  |  | 01 - November - 2014 |
| ARO. RAMP. 130 Categorization of Findings | 33 |  |  | 01 - November - 2014 |
| ARO. RAMP. 135 Follow - up Actions on Findings | 33 |  |  | 01 - November - 2014 |
| ARO. RAMP. 140 Grounding of Aircraft | 34 |  |  | 01 - November - 2014 |
| ARO. RAMP. 145 Reporting | 34 |  |  | 01 - November - 2014 |
| ARO. RAMP. 150 Agency ( EASA ) Coordination Tasks | 35 |  |  | 01 - November - 2014 |
| ARO. RAMP. 155 Annual Report | 35 |  |  | 01 - November - 2014 |
| ARO. RAMP. 160 Information to the Public and Protection of Information | 35 |  |  | 01 - November - 2014 |
| GUIDANCE MATERIAL & ACCEPTABLE MEANS of COMPLIANCE | 37 |  | 106 | 01 - November - 2014 |
| Appendix I. | 107 |  | 108 | 01 - November - 2014 |
| Appendix II. | 109 |  | 110 | 01 - November - 2014 |
| Appendix III. | 111 |  | 112 | 01 - November - 2014 |
| Appendix IV. | 113 |  | 116 | 01 - November - 2014 |
| Appendix V. | 117 |  | 118 | 01 - November - 2014 |
| Appendix VI. | 119 |  | 120 | 01 - November - 2014 |

ARO. GEN. 005   Scope

This  Annex  establishes  requirements  for  the  administration  and  management  system  to  be fulfilled  by  the  GDCA  of  RA  with  cooperation  by  EASA  for  the  implementation  and  enforcement  of  Air  CR - EC  No 216 / 2008  and  its  Implementing  Rules  regarding  Civil  Aviation  Air  Operations  as  the  “ ARM - AIR  OPERATIONS “  regulations.

INTENTIONALLY            LEFT            BLANK

SUBPART  GEN.     GENERAL    REQUIREMENTS

SECTION  I.     General

ARO. GEN. 115   Oversight  Documentation

The  Competent  Authority  ( GDCA )  shall  provide  all  legislative  acts,  standards,  rules, technical  publications  and  related  documents  to  relevant  personnel  in  order  to  allow  them  to  perform  their  tasks  and  to  discharge  their  responsibilities.

ARO. GEN. 120   Means  of  Compliance

a )   The  GDCA  shall  develop  Acceptable  Means  of  Compliance  ( AMC )  that  may  be  used  to  establish  compliance  with  Air  Operation  CR - EC  No  216 / 2008  and  its Implementing  Rules.  When  the  AMC  are  complied  with,  the  related  requirements  of  the  Implementing  Rules  are  met ;

b )   Alternative  Means  of  Compliance  may  be  used  to  establish  compliance  with  the Implementing  Rules ;

c )   The  GDCA  shall  establish  a  system  to  consistently  evaluate  that  all  Alternative  Means  of  Compliance  used  by  itself  or  by  organisations  and  persons  under  its  oversight  allow  the  establishment  of  compliance  with  Air  Operation  Regulation  and  its  Implementing  Rules ;

d )   The  GDCA  shall  evaluate  all  alternative  means  of  compliance  proposed  by  an  organisation  in  accordance  with  ORO. GEN. 120  ( b )  by  analysing  the  documentation  provided  and,  if  considered  necessary,  conducting  an  inspection  of  the  organisation.

When  the  GDCA  finds  that  the  Alternative  Means  of  Compliance  are  in  accordance  with  the  Implementing  Rules,  it  shall  without  undue  delay :

1 )   notify  the  applicant  that  the  alternative  means  of  compliance  may  be  implemented  and,  if applicable,  amend  the  approval  specialised  operation  authorisation  or  certificate  of  the  applicant  accordingly ;                          and

e )   When  the  GDCA  itself  uses  Alternative  Means  of  Compliance  to  achieve  compliance  with  Air  Operation   Regulation  and  its  Implementing  Rules  it  shall  make  them  available  to  all  organisations  and  persons  under  its  oversight ;

The  GDCA  shall  provide  a  full  description  of  the  Alternative  Means  of  Compliance,  including  any  revisions  to  procedures  that  may  be  relevant,  as  well  as  an assessment  demonstrating  that  the  Implementing Rules  are  met.

ARO. GEN. 125   Information  to  the  Agency

The  GDCA  as  a  Competent  Authority  use  the  partership  procedure  shall  provide  the  Agency  with  safety - significant  information  stemming  from  the  occurrence  reports  it  has  received.

ARO. GEN. 135   Immediate  Reaction  to  a  Safety  Problem

a )   The  GDCA  shall  implement  a  system  to  appropriately  collect,  analyse  and  disseminate  safety  information ;

b )   The  Agency  shall  implement  a  system  to  appropriately  analyse  any  relevant  safety information  received  and  without  undue  delay  provide  to  Member  States  any  information,  including  recommendations  or  corrective  actions  to  be  taken,  necessary  for  them  to  react  in a  timely  manner  to  a  safety  problem  involving  products,  parts,  appliances,  persons  or  organisations  subject  to  CR - EC  No 216/2008   and  its  Implementing  Rules ;

c )   Upon  receiving  the  information  referred  to  in  the  ( b ),  the  GDCA  shall  take  adequate  measures  to  address  the  safety  problem ;

d )   Measures  taken  under  ( c )  shall  immediately  be  notified  to  all  persons  or  organisations which  need  to  comply  with  them  under  Air  Operations  Regulation  and  its  Implementing  Rules.  The  GDCA  as  a  Competent  Authority  shall  also  notify  those   measures  to  the  Agency.

SECTION  II.     Management

ARO. GEN. 200   Management  System

a )   The   GDCA  shall  establish  and  maintain  a  management  system,  including  as  a minimum :

1 )   Documented  policies  and  procedures  to  describe  its  organisation,  means  and  methods  to  achieve  compliance  with  Air  Operation  Regulation  and  its  Implementing  Rules.  The  procedures  shall  be  kept  up  to  date  and  serve  as  the  basic  working  documents  within  GDCA  that competent  authority  for  all  related  tasks ;

2 )   A  sufficient  number  of  personnel  to  perform  its  tasks  and  discharge  its  responsibilities.  Such  personnel  shall  be  qualified  to  perform  their  allocated  tasks  and  have  the  necessary  knowledge,  experience,  initial  and  recurrent  training  to  ensure  continuing  competence.                       A  system  shall  be  in  place  to  plan  the  availability  of  personnel,  in  order  to  ensure  the  proper  completion  of  all  tasks ;

3 )   Adequate  facilities  and  office  accommodation  to  perform  the  allocated  tasks ;

4 )   A  function  to  monitor  compliance  of  the  management  system  with  the  relevant requirements  and  adequacy  of  the  procedures  including  the  establishment  of  an  internal  audit  process  and  a  safety  risk  management  process.  Compliance  monitoring  shall  include  a  feedback  system  of  audit  findings  to  the  senior  management  of  the  GDCA  to  ensure  implementation  of corrective  actions as  necessary ;                      and

5 )   A  person  or  group  of  persons,  ultimately  responsible  to  the  senior  management of  the  GDCA  for  the  compliance  monitoring  function.

b )   The  GDCA  shall,  for  each  field  of  activity,  including  management  system,  appoint  one  or  more  persons  with  the  overall  responsibility  for  the  management  of  the  relevant  task(s) ;

c )   The  GDCA  shall  establish  procedures  for  participation  in  a  mutual  exchange  of  all  necessary  information  and  assistance  with  other  competent  authorities  concerned  including  on  all  findings  raised  and  follow - up  actions  taken  as  a  result  of  oversight  of  persons  and  organisations  exercising  activities,  but  certified  or  authorised  by  or  making  declarations  to  the  Competent  Authority .

ARO. GEN. 205   Allocation  of  Tasks  to  Qualified  Entities

a )   Tasks  related  to  the  initial  certification  specialized  operation  authorization  or  continuing  oversight  of  persons  or  organisations  subject  to  Air  Operations  Regulation  and  its Implementing  Rules  shall  be  allocated  by  GDCA    only  to  qualified  entities. When  allocating  tasks,  the  GDCA  shall  ensure  that  it  has :

1 )   Put  a  system  in  place  to  initially  and  continuously  assess  that  the  qualified  entity  complies  with  Annex  V  ( Part - SPA )  to  Air  Operations  Regulation.  This  system  and  the  results  of  the  assessments  shall  be  documented.

2 )   Established  a  documented  agreement  with  the  qualified  entity, approved  by  both  parties  at  the  appropriate  management  level,  which  clearly  defines :

( i )      the  tasks  to  be  performed ;

( ii )     the  declarations,  reports  and  records  to  be  provided ;

( iii )    the  technical  conditions  to  be  met  in  performing  such  tasks ;

( iv )    the  related  liability  coverage ;                                                                     and

( v )     the  protection  given  to  information  acquired  in  carrying  out  such  tasks.

b )   The  GDCA  shall  ensure  that  the  internal  audit  process  and  safety  risk  management  process  required  by  ARO. GEN. 200  ( a )( 4 )  covers  all  certification  authorization  or  continuing  oversight  tasks  performed  on  its  behalf.

ARO. GEN. 210   Changes  in  the  Management  System

a )   The  GDCA  shall  have  a  system  in  place  to  identify  changes  that  affect  its  capability  to  perform  its  tasks  and  discharge  its  responsibilities  as  defined  in  Air  Operations  Regulation   and  its  Implementing  Rules. This  system  shall  enable  it  to  take  action  as  appropriate  to  ensure  that  its  management  system  remains  adequate  and  effective.

b )   The  GDCA  shall  update  its  management  system  to  reflect any  change  to  Air  Operations  Regulation  and  its  Implementing Rules  in  a  timely  manner , so  as  to  ensure  effective  implementation.

ARO. GEN. 220   Record - Keeping

a )   The  GDCA  shall  establish  a  system  of  record - keeping  providing  for  adequate  storage,  accessibility  and  reliable  traceability  of :

1 )     the  management  system’s  documented  policies  and  procedures ;

2 )     training,  qualification  and  authorisation  of   its  personnel ;

3 )     the  allocation  of  tasks,  covering  the  elements  required  by  ARO. GEN. 205  as  well  as  the  details  of  tasks  allocated ;

4 )     certification  processes  and  continuing  oversight  of  certified  organisations ;

4 a )   the  process  of  authorization  of  a  high  risk  commercial  specialized  operation  and

continuing  oversight  of  an  authorization  holder ;

5 )     declaration  processes  and  continuing  oversight  of  declared  organisations ;

6 )     details  of  training  courses  provided  by  certified  organisations,  and  if  applicable,  records  relating  to  FSTD’s  used  for  such  training ;

7 )     oversight  of  persons  and  organizations  exercising  activities  within  the  territory  of  the Member  State,  but  overseen,  certified  or  authorized  by  the  competent  authority  of  another Member  State  or  the  Agency,  as  agreed  between  these  authorities ;

8 )     oversight  of  operations  of  other - than - complex  motor - powered  aircraft  by  non -commercial  operators ;

9 )     the  evaluation  and  notification  to  the  Agency  of  alternative  means  of  compliance  proposed  by  organizations  subject  to  certification  or  authorization  and  the  assessment  of  alternative  means  of  compliance  used  by  the  competent  authority  itself ;

10 )    findings,  corrective  actions  and  date  of  action closure ;

11 )    enforcement  measures  taken ;

12 )    safety  information  and  follow-up  measures ;                                      and

13 )    the  use  of  flexibility  provisions  in  accordance  with  Article 14  of  Air Operations

CR - EC  No 216 / 2008.

b )   The  Competent  Authority  shall  maintain  a  list  of  all  organization  certificates  and  specialized operations  authorizations  it  issued  as  well  as  declarations  it  received.

c )   All  records  shall  be  kept  for  the  minimum  period  specified  in  this  Regulation.                                   In  the  absence  of  such  indication,  records  shall  be  kept  for  a  minimum  period  of  5  ( five )  years  subject  to  applicable  data  protection  law.

ARO. OPS. 230     Determination   of   Disruptive   Schedules

For  the  purpose  of  Flight  Time  Limitations,  the  Competent  Authority  shall  determine,  in accordance  with  the  Definitions  of   “ Early  Type ”  and   “ Late  Type ”   of   disruptive  schedules in  point  ORO. FTL. 105   of  Annex  III,  which  of  those  two  types  of  disruptive  schedules  shall apply  to  all  CAT  operators  under  its  oversight.

ARO. OPS. 235     Approval   of   Individual   Flight   Time   Specification   Schemes

a )    The  Competent  Authority  shall  approve  flight  time  specification  schemes  proposed  by CAT  Operators  if  the  Operator  demonstrates  compliance  with  CR - EC No  216 / 2008  and  Subpart  FTL  of  Annex  III  to  this  Regulation ;

b )    Whenever  a  flight  time  specification  scheme  proposed  by  an  Operator  deviates  from  the applicable  certification  specifications  issued  by  the  Agency,  the  Competent  Authority  shall apply  the  procedure  described  in  Article  22 ( 2 )  of  CR - EC  No  216 / 2008 ;

c )    Whenever  a  flight  time  specification  scheme  proposed  by  an  Operator   derogates  from applicable  implementing  rules,  the  Competent  Authority  shall  apply  the  procedure  described  in Article  14 ( 6 )  of  CR - EC No  216 / 2008 ;

d )    Approved  deviations  or  derogations  shall  be  subject,  after  being  applied,  to  an  assessment to  determine  whether  such  deviations  or  derogations  should  be  confirmed  or  amended.                         The  Competent  Authority  shall  conduct  an  independent  assessment  based  on  information provided  by  the  Operator.  The  assessment  shall  be  proportionate,  transparent  and  based  on scientific  principles  and   knowledge.

INTENTIONALLY            LEFT            BLANK

SECTION  III    Oversight,  Certification  and  Enforcement

ARO. GEN. 300    Oversight

a )    The  GDCA  shall  verify :

1 )   compliance  with  the  requirements  applicable  to  organizations  or  type  of  operations  prior

to  the  issue  of  a  certificate,  approval  or  authorization,  as  applicable ;

2 )   continued  compliance  with  the  applicable  requirements  of  organizations  it  has  certified,

specialized  operations  it  has  authorized  and  organizations  from  whom  it  received  a

declaration ;

3 )   continued  compliance with  the  applicable requirements of  non- commercial operators of

other-than-complex  motor-powered  aircraft ;               and

4 )    implementation  of  appropriate  safety  measures  mandated  by  the  GDCA  as  defined  in

ARO. GEN. 135 ( c )  and  ( d ).

b )   This  verification  shall :

1 )    be  supported  by  documentation  specifically  intended  to  provide  personnel  responsible

for  safety  oversight  with  guidance  to  perform  their  functions ;                                                                      2 )    provide  the  persons  and  organisations  concerned  with  the  results  of  safety  oversight

activity ;                                                                                                                                                                          3 )    be  based  on  audits  and  inspections,  including  ramp  and  unannounced  inspections ;  and           4 )    provide  the  GDCA  with  the  evidence  needed  in  case  further  action  is  required,

including  the  measures  foreseen  by  ARO. GEN. 350  and  ARO. GEN. 355.

c )   The  scope of  oversight defined  in  ( a )  and  ( b ) shall take  into  account  the  results  of  past oversight  activities  and  the  safety  priorities ;

d )   Without  prejudice  to  the  competences  of  the  State  and  to  their  obligations  as  set  out  in ARO. RAMP,  the  scope  of  the  oversight  of  activities  performed  in  the  territory of  a  State  by persons  or  organisations established  or  residing  in  another  State  shall  be  determined  on  the

basis  of  the  safety  priorities, as  well  as  of  past  oversight  activities ;

e  )   The  GDCA  shall  collect  and  process  any  information  deemed  useful  for  oversight,  including  for  ramp  and  unannounced  inspections.

ARO. GEN. 305   Oversight  Programme

a )   The  GDCA  shall  establish  and  maintain  an  oversight  programme  covering  the  oversight  activities  required  by  ARO. GEN. 300  and  by  ARO. RAMP.

b )   For  organisations  certified  by  the  GDCA,  the  oversight  programme  shall  be  developed  taking  into  account  the  specific  nature  of  the  organisation,  the  complexity  of  its  activities,  the  results  of  past  certification  and / or  oversight  activities  required  by   ARO. GEN   and   ARO. RAMP  and  shall  be  based  on  the  assessment  of  associated  risks.                                                                              It  shall  include  within  each  oversight  planning  cycle :

1 )   audits  and  inspections,  including  ramp  and  unannounced  inspections  as  appropriate ;   and

2 )   meetings  convened  between  the  Accountable  Manager  and  the  GDCA  to  ensure  both  remain  informed  of  significant  issues.

c )   For  organisations  certified  by  the  GDCA  an  oversight  planning  cycle  not  exceeding  24  months  shall  be  applied.  The  oversight  planning  cycle  may  be  reduced  if  there  is  evidence  that  the  safety  performance  of  the  organisation  has  decreased.

The  oversight  planning  cycle  may  be  extended  to  a  maximum  of  36  months  if  the  GDCA  has  established  that,  during  the  previous  24  months :

1 )    the  organisation  has  demonstrated  an  effective  identification  of  aviation  safety  hazards  and  management  of  associated  risks ;                                                                                                                       2 )    the  organisation  has  continuously  demonstrated  under  ORO. GEN. 130   that  it  has  full control  over  all  changes ;

3 )    no  Level 1  findings  have  been i ssued ;                            and                                                              4 )    all  corrective  actions  have  been  implemented  within  the  time  period  accepted  or  extended  by  the  GDCA  as  defined  in  ARO. GEN. 350 ( d )( 2 ).                                                                             The  oversight  planning  cycle  may  be  further  extended  to  a  maximum  of  48  months  if,  in  addition  to  the  above,  the  organisation  has  established,  and  the  GDCA  has  approved,  an  effective  continuous  reporting  system  to  the  GDCA  on  the  safety  performance  and  regulatory  compliance  of  the  organisation  itself ;

d )   For  organizations  declaring  their  activity  to  the  GDCA,  the  oversight programme  shall  be  based  on  the  specific  nature  of  the  organization,  the  complexity  of  its activities  and  the  data  of  past  oversight  activities  and  the  assessment  of  risks  associated  with the  type  of  activity  carried  out.   It  shall  include  audits  and  inspections,  including  ramp  and unannounced  inspections,  as  appropriate ;

d 1 )   For  organizations  holding  a  specialized  operations  authorization,  the  oversight  programme shall  be  established  in  accordance  with  ( d )  and  shall  also  take  into  account  the  past  and current  authorization   process  and  the  validity  period  of  the  authorization.

e )   For  persons  holding  a  licence,  certificate,  rating,  or  attestation  issued  by  the  Competent Authority  the  oversight  programme  shall  include  inspections,  including  unannounced  inspections,  as  appropriate.

f )   The  oversight  programme  shall  include  records  of  the  dates  when  audits,  inspections  and meetings  are  due  and  when  such  audits,  inspections  and  meetings  have  been  carried  out.

ARO. GEN. 310     Initial  Certification  Procedure  —  Organisation’s

a )   Upon  receiving an  application for  the  initial  issue  of  a  certificate f or  an  organisation, the  GDCA  shall  verify  the  organization’s compliance with  the  applicable requirements.  This verification  may  take  into  account  the  statement  referred  to  in  ORO. AOC. 100 ( b ) ;

b )   When  satisfied  that  the  organization  is  in  compliance  with  the  applicable  requirements, the GDCA  shall  issue  the  certificate(s), as  established  in  Appendices  I  and  II.  The  certificate(s) shall  be  issued  for  an  unlimited  duration.  The  privileges  and  scope  of  the  activities  that  the organization  is  approved  to  conduct  shall  be  specified  in  the  terms  of  approval  attached  to  the certificate(s) ;

c )   To  enable  an  organization  to  implement  changes without  prior  GDCA  of  RA  approval  in accordance  with  ORO. GEN. 130,  the  GDCA  shall  approve  the  procedure  submitted  by  the organization  defining  the  scope  of  such  changes  and  describing  how  such  changes  will  be managed  and  notified.

ARO. GEN. 330     Changes  —  Organisation’s

a )   Upon  receiving an  application  for  a change  that  requires  prior  approval,  the  GDCA  shall verify  the  organisation’s  compliance  with  the  applicable  requirements  before  issuing  the approval. The  GDCA  shall  prescribe  the  conditions  under  which  the  organization  may  operate during  the  change, unless  the  GDCA  determines  that  the  organization’s  certificate  needs  to  be suspended.

When  satisfied  that  the  organization  is  in  compliance  with  the  applicable requirements, the GDCA  shall  approve  the  change ;

b )   Without  prejudice  to  any  additional  enforcement  measures, when  the  organization implements  changes  requiring  prior  approval  without  having  received   GDCA  approval  as defined  in  ( a ),  the  GDCA  shall  suspend,  limit  or  revoke  the  organization’s  certificate ;

c )   For  changes  not  requiring  prior  approval,  the  GDCA  shall  assess  the  information  provided in  the  notification  sent  by  the  organization  in  accordance  with  ORO. GEN. 130  to  verify compliance  with  the  applicable  requirements. In  case  of  any  non-compliance,  the  GDCA  shall :

1 )   notify  the  organisation  about  the  non-compliance and  request  further  changes ;

2 )   in  case  of  Level 1  or  2  Findings,  act  in  accordance  with   ARO. GEN. 350.

ARO. GEN. 345    Declaration  -  Organisation’s

a )   Upon  receiving  a  declaration  from  an  organization  carrying  out  or  intending  to  carry  out  activities  for  which  a  declaration  is  required,  the  GDCA  shall verify  that  the  declaration  contains  all  the  information  required  by  Part - ORO  and  shall  acknowledge  receipt  of  the  declaration  to  the  organisation ;

b )   If  the  declaration  does  not  contain  the  required  information,  or  contains  information  that  indicates  non - compliance  with  applicable  requirements,  the  GDCA  shall  notify  the  organisation  about  the  non - compliance  and  request  further  information.  If  deemed  necessary  the  GDCA  shall  carry  out  an  inspection  of  the organisation.                                                                                If  the  non - compliance  is  confirmed,  the  GDCA  shall  take  action  as  defined  in  ARO. GEN. 350.

ARO.GEN.350   Findings  and  Corrective  Actions  -  Organisation’s

a )   The  GDCA  for  oversight  in  accordance  with  ARO. GEN. 300 ( a ) shall  have  a  system  to  analyse  findings  for  their  safety  significance ;

b )  A  Level  1  Finding  shall  be  issued  by  the  Competent  Authority  when  any  significant  non -compliance  is  detected  with  the  applicable  requirements  of  CR - EC No  216 / 2008  and  its Implementing  Rules,   with  the  organization’s  procedures  and  manuals  or  with  the  terms  of  an approval,  certificate,  specialized   operation  authorization  or  with  the  content  of  a  declaration which  lowers  safety  or  seriously  hazards  flight  safety.

The  Level  1  Findings  shall  include :

1 )    failure  to  give  the  GDCA  access  to  the  organisation’s facilities  as  defined  in  ORO. GEN. 140  during  normal  operating  hours  and  after  two  written  requests ;

2 )    obtaining  or  maintaining  the  validity  of  the  organisation  certificate  or  specialized  operations  authorization  by  falsification  of  submitted  documentary  evidence ;

3 )    evidence  of  malpractice  or  fraudulent  use  of  the  organisation  certificate  or  specialized  operations  authorization ;           and

4 )    the  lack  of  an  Accountable  Manager.

c )   A  Level  2  Finding  shall  be  issued  by  the  Competent  Authority  when  any  non - compliance is  detected  with  the  applicable  requirements  of  CR - EC No 216 / 2008  and  its  Implementing Rules,  with  the  organization’s  procedures  and  manuals  or  with  the  terms  of  an  approval, certificate,  specialized  operation  authorization  or  with  the  content  of  a  declaration  which  could lower  safety  or  hazard  flight  safety ;

d )   When  a  finding  is  detected  during  oversight  or  by  any  other  means,  the  GDCA  shall,  without  prejudice  to  any  additional  action  required  by  Air Operations  Regulation  and  its  Implementing  Rules,  communicate  the  finding  to  the  organisation  in  writing  and  request  corrective  action  to  address  the  non-compliance(s)  identified.  Where  relevant,  the  GDCA  shall inform  the  State  in  which  the  aircraft  is  registered ;

1 )    in  the  case  of  Level 1  Findings  the  GDCA  shall  take  immediate  and  appropriate  action  to  prohibit  or  limit  activities,  and  if  appropriate,  it  shall  take  action  to  revoke  the  certificate  specialized  operations  authorization  or  specific  approval  or  to  limit  or  suspend  it  in  whole  or  in  part,  depending  upon  the  extent  of  the  Level 1  finding,  until  successful  corrective  action  has  been  taken  by  the  organisation ;

2 )    in  the  case  of  Level 2  Findings,  the  GDCA  shall :

( i )   grant  the  organisation  a  corrective  action  implementation  period  appropriate to  the  nature  of  the  finding  that  in  any  case  initially  shall  not  be  more  than  3 ( three )  months.  At  the  end  of  this  period,  and  subject  to  the  nature  of  the  finding,  the  GDCA  may  extend  the  3 ( three -month )  period  subject  to  a  satisfactory  corrective  action  plan  agreed  by  the  GDCA ;      and

( ii )  assess  the  corrective  action  and  implementation  plan  proposed  by  the  organisation  and,  if  the  assessment  concludes  that  they  are  sufficient  to address  the  non-compliance(s),  accept  these.

3 )    where  an  organisation  fails  to  submit  an  acceptable  corrective  action  plan,  or  to  perform  the  corrective  action  within  the  time  period  accepted  or  extended  by  the GDCA,  the  finding  shall  be  raised  to  a  Level 1  Finding  and  action  taken  as  laid  down  in  ( d )( 1 ) ;

4 )    the  GDCA  shall  record  all  findings  it  has  raised  or  that  have  been  communicated  to  it  and,  where  applicable,  the  enforcement  measures  it  has  applied,  as  well  as  all  corrective  actions  and  date  of  action  closure  for  findings.

e )   Without  prejudice  to  any  additional  enforcement   measures,  when  the  authority  of  a Member  State  acting  under  the  provisions  of  ARO. GEN. 300 ;

f  )  Iidentifies  any  non - compliance  with  the  applicable  requirements  of  CR - EC No 216 / 2008 and  its  Implementing  Rules  by  an  organization  certified  by  or  authorized  by  or  declaring  its activity  to  the  competent  authority  of  another  Member  State  or  the  Agency,  it  shall  inform  that  competent  authority  and  provide  an  indication  of  the  level  of  finding.

ARO. GEN. 355    Findings  and  Enforcement  Measures  -  Persons

a )   If,  during  oversight  or  by  any  other  means,  evidence  is  found  by  the  GDCA  responsible  for  oversight  in  accordance  with  ARO. GEN. 300 ( a )  that  shows  a  non-compliance with  the  applicable  requirements  by  a  person  holding  a  licence,  certificate,  rating  or  attestation  issued  in  accordance  with  CR - EC No 216 / 2008  and  its  Implementing  Rules,  the  GDCA  shall  act  in  accordance  with  ARA. GEN. 355( a )  to  ( d ) of  Annex VI  ( Part - ARA ) to  ARM - AIR  CREW ; b )   If,  during  oversight  or  by  any  other  means,  evidence  is  found  showing  a  non - compliance  with  the  applicable  requirements  by  a  person  subject  to  the  requirements  laid  down  in  Air  Operations  Regulation  and  its  Implementing  Rules and  not  holding  a  licence,  certificate,  rating  or  attestation  issued  in  accordance  with  that  Regulation  and  its  Implementing  Rules,  the  GDCA  that  identified  the  non - compliance  shall  take  any  enforcement  measures  necessary  to prevent  the  continuation  of  that  non - compliance.

ARO. GEN. 360      Findings  and  Enforcement  Measures  —  all  Operator’s

If,  during  oversight  or  by  any  other  means,  evidence  is  found  showing  a  non-compliance  with the  applicable  requirements  by  an  Operator  subject  to  the  requirements  laid  down  in  CR - EC No 216 / 2008  and  its  Implementing  Rules,  the  GDCA  that  identified  the  non -compliance  shall  take  any  enforcement  measures  necessary  to  prevent  the  continuation  of  that  non - compliance.

SUBPART  OPS.     AIR   OPERATION’S

SECTION  I.     Certification  of  Commercial  Air  Transport  Operators

ARO. OPS. 100   Issue  of  the  Air  Operator  Certificate

a )   The  GDCA  shall  issue  the  Air  Operator  Certificate  ( AOC )  when  satisfied  that  the  operator  has  demonstrated  compliance  with  the  elements  required  in  Annex  III,  Part - ORO, ORO. AOC. 100 ;

b )   The  certificate  shall  include  the  associated  operations  specifications.

c )   The  GDCA  may  determine  specific  operational  limitations.                                                                   Such  limitations shall  be  documented  in  the  Operations  Specifications.

ARO. OPS. 105   Code - Share  Arrangements

In  considering  the  safety  of  a  code - share  agreement  involving  a  third - country  operator,  the  GDCA  shall :

1 )    satisfy  itself,  following  the  verification  by  the  operator  as  set  out  in  ORO. AOC. 115,  that  the  third - country  operator  complies  with  the  applicable  ICAO  standards ;

2 )    liaise  with  the  competent  authority  of  the  State  of  the  third-country  operator  as  necessary.

ARO. OPS. 110   Lease  Agreements

a )   The  GDCA  shall  approve  a  lease  agreement  when  satisfied  that  the  Operator  certified  in  accordance  with  Annex  III  ( Part - ORO )  complies  with :

1 )    ORO. AOC. 110 ( d ),  for  dry  leased - in  third  country  aircraft ;

2 )    ORO. AOC. 110 ( c ),  for  wet  lease - in  of  an  aircraft  from  a  third  country  operator ;

3 )    ORO. AOC. 110 ( e ),  for  dry  lease - out  of  an  aircraft  to  any  operator ;

4 )    relevant  requirements  of  continuing  airworthiness  and  air  operations,  for  dry  lease - in  of  an  aircraft  registered  in  the  RA  and  wet  lease - in  of  an  aircraft  from  an  EU  operator.

b )   The  approval  of  a  wet  lease - in  agreement  shall  be  suspended  or  revoked  whenever :

1 )    the  AOC  of  the  lessor  or  lessee  is  suspended  or  revoked ;

2 )    the  lessor  is  subject  to  an  operating  ban  pursuant  to  CR - EC No  2111 / 2005  of  the  European  Parliament  and  of  the  Council.

c )   The  approval  of  a  dry  lease-in  agreement  shall  be  suspended  or  revoked  whenever  the  certificate  of  airworthiness  of  the  aircraft  is  suspended  or  revoked ;

d )   When  asked  for  the  prior  approval  of  a  dry - lease  out  agreement  in  accordance  with  ORO. AOC. 110 ( e ),  the  GDCA  of  RA  shall  ensure :

1 )    proper  coordination  with  the  Competent  Authority  responsible  for  the  continuing  oversight  of  the  aircraft,  in  accordance  with  CR - EC No 2042 / 2003  (  if  it  is  not  the  same  authority ),  or  for  the  operation  of  the  aircraft,  if  it  is  not  the  same  Authority ;

2 )    that the  aircraft  is  timely  removed  from  the  operator’s  AOC.

INTENTIONALLY            LEFT            BLANK

SECTION  I  a    Authorization  of  High  Risk  Commercial  Specialized  Operations

ARO. OPS. 150    Authorization  of  High  Risk  Commercial  Specialized  Operations

a )   Upon  receiving  an  application  for  the  issue  of  a  high  risk  commercial  specialized  operations  authorization,  the  Competent  Authority  of  the  Operator  shall  review  the  operator’s risk  assessment  documentation  and  Standard  Operating  Procedures  ( SOP ),  related  to  one  or more  planned  operations  and  developed  in  accordance  with  the  relevant  requirements  of  Annex VIII  ( Part - SPO ) ;

b )   When  satisfied  with  the  risk  assessment  and  SOP,  the  Competent  Authority  of  the  Operator  shall  issue  the  authorization,  as  established  in  Appendix  VI.  The  authorization  may  be  issued  for  a  limited  or  an  unlimited  duration.  The  conditions  under  which  an  Operator  is authorized  to  conduct  one  or  more  high  risk  commercial  specialized  operations  shall  be specified  in  the  authorization ;

c )   Upon  receiving  an  application  for  a  change  to  the  authorization,  the  Competent  Authority  of  the  Operator  shall  comply  with  ( a )  and  ( b ).  It  shall  prescribe  the  conditions  under  which the  Operator  may  operate  during  the  change,  unless  the  Competent  Authority  determines  that the  authorization  needs  to  be  suspended ;

d )   Upon  receiving  an  application  for  the  renewal  of  the  authorization,  the  Competent  Authority  of  the  Operator   shall  comply  with  ( a )  and  ( b ).  It  may  take  into  account  the  past authorization  process  and  oversight  activities ;

e )   Without  prejudice  to  any  additional  enforcement  measures,  when  the  Operator  implements changes  without  having  submitted  an  amended  risk  assessment  and  SOP,  the  Competent Authority  of  the  Operator  shall   suspend,  limit  or  revoke  the  authorization ;

f )   Upon  receiving  an  application  for  the  issue  of  an  authorization  for  a  cross-border  high  risk commercial  specialized  operation,  the  Competent  Authority  of  the  Operator  shall  review  the operator’s  risk  assessment  documentation  and  Standard  Operating  Procedures  ( SOP )  in coordination  with  the  competent  authority  of  the  place  where  the  operation  is  planned  to  be conducted.  When  both  authorities  are  satisfied  with  the  risk  assessment  and  SOP,  the Competent  Authority  of  the  Operator  shall  issue  the  authorization.

ARO. OPS. 155     Lease  Agreements

a )   The  Competent  Authority  shall  approve  a  lease  agreement  involving  a  third  country registered  aircraft  or  a  third  country  operator  when  the  SPO  operator  has  demonstrated compliance  with  ORO. SPO. 100 ;

b )   The  approval  of  a  dry  lease - in  agreement  shall  be  suspended  or  revoked  whenever  the certificate  of  airworthiness  of  the  aircraft  is  suspended  or  revoked.

INTENTIONALLY          LEFT            BLANK

SECTION  II.   Approvals

ARO. OPS. 200   Specific  Approval  Procedure

a )   Upon  receiving  an  application  for  the  issue  of  a  specific  approval  or  changes  thereof, the  GDCA  shall  assess  the  application  in  accordance  with  the relevant  requirements  of  Annex V            ( Part - SPA )  and  conduct,  where  relevant,  an  appropriate  inspection  of  the  Operator ;

b )   When  satisfied  that  the  Operator  has  demonstrated  compliance  with  the  applicable requirements,  the  GDCA  shall  issue  or  amend  the  approval.                                                                               The  approval  shall  be  specified  in :

1 )   the  Operations  Specifications,  as  established   in  this  Appendix  II,  for  Commercial  Air  Transport  Operations ;                              or

2 )   the  list  of  specific  approvals,  as  established  in  Appendix  V,  for  non - commercial  operations  and  specialized  operations.

ARO. OPS. 205   Minimum  Equipment  List  Approval

a )   When  receiving  an  application  for  initial  approval  of  a  Minimum  Equipment  List  ( MEL ) or  an  amendment  thereof  from  an  operator,  the  GDCA  shall  assess each  item  affected,  to  verify  compliance  with  the  applicable  requirements,  before  issuing  the  approval ;

b )   The  GDCA  shall  approve  the  Operator’s  procedure  for  the  extension  of  the  applicable  rectification  intervals  B,  C  and  D,  if  the  conditions  specified  in  ORO. MLR. 105 ( f )  are  demonstrated  by  the  Operator  and  verified  by  the  GDCA ;

c )   The  GDCA  shall  approve,  on  a  case - by - ase  basis,  the  operation  of  an  aircraft  outside  the  constraints  of  the  MEL  but  within  the  constraints  of  the  Master  Minimum  Equipment  List  ( MMEL ),  if  the  conditions  specified  in  ORO. MLR. 105  are  demonstrated  by  the  Operator  and  verified  by  the  GDCA.

ARO. OPS. 210   Determination  of  Distance  or  Local  Area

The  GDCA  of  RA  may  determine  a  a  distance  or  local  area  for  the  purpose  of  operations.

ARO. OPS. 215   Approval  of  Helicopter  Operations  Over  a  Hostile  Environment  Located

Outside  a  Congested  Area

a )   The  GDCA  shall  designate  those  areas  where  helicopter  operations  may  be  conducted  without  an  assured  safe  forced  landing  capability,  as  described  in  CAT. POL. H. 420 ;

b )   Before  issuing  the  approval  referred  to  in  CAT. POL. H. 420  the  GDCA  shall  have  considered  the  Operator’s  substantiation  precluding  the  use  of  the  appropriate  performance  criteria.

ARO. OPS. 220   Approval  of  Helicopter  Operations  to  or  from  a  Public  Interest  Site

The  approval  referred  to  in  CAT. POL. H. 225  shall  include  a  list  of  the  public  interest  site(s)  specified  by  the  Operator  to  which  the  approval  applies.

ARO. OPS. 225   Approval  of  Operations  to  an  Isolated  Aerodrome

The  approval  referred  to  in  CAT. OP. MPA. 106  shall  include  a  list  of  the  aerodromes  specified  by  the  Operator  to  which  the  approval  applies.

SECTION  III     Oversight  of  Operations

ARO. OPS. 300     Introductory  Flights

The  GDCA  of  RA  may  establish  additional  conditions  for  introductory  flights  carried  out  in  accordance  with  Part - NCO  in  the  territory  of  the  State.                                                                           Such  conditions  shall  ensure  safe  operations  and  be  proportionate.

INTENTIONALLY          LEFT            BLANK

SUBPART   RAMP.     RAMP  INSPECTIONS  of  AIRCRAFT  of  OPERATORS  under  the

REGULATORY  OVERSIGHT  of  another  STATE

ARO. RAMP. 005   Scope

This  Subpart  establishes  the  requirements  to  be  followed  by  the  GDCA  of  RA  when  exercising  its  tasks  and  responsibilities  regarding  the  performance  of  Ramp  Inspections  of  aircraft  used  by  third  country  operators  when  landed  at  aerodromes  located  in  the  territory  of Republic  of  Armenia.

ARO. RAMP. 100   General

a )   Aircraft,  as  well  as  their  crew,  shall  be  inspected  against  the  applicable  requirements ;

b )   In  addition  to  conducting  Ramp  Inspections  included  in  its  oversight  programme  established  in  accordance  with  ARO. GEN. 305,  the  GDCA  shall  perform  a  Ramp  Inspection  of  an  aircraft  suspected  of  not  being  compliant  with  the  applicable  requirements ;

c )   Within  the  development  of  the  oversight  programme  established  in  accordance  with  ARO. GEN. 305,  the  GDCA  shall  establish  an  annual  programme  for  the  conduct  of  ramp  inspections  of  aircraft.

This  programme  shall :

1 )    be  based  on  a  calculation  methodology  that  takes  into  account  historical  information  on  the  number  and  nature  of  Operators  and  their  number  of  landings  at  its  aerodromes,  as  well  as  safety  risks ;                                                              and

2 )    enable  the  GDCA  to  give  priority  to  the  inspections  of  aircraft  on  the  basis  of  the  list  referred  to  in  ARO. RAMP. 105 ( a ).

d )   When  it  so  deems  necessary,  the  GDCA,  shall  conduct  Ramp  Inspections  of  aircraft  to  verify  compliance  with  the  applicable  requirements  for  the  purpose  of :

1 )    certification  tasks  assigned  to  the  GDCA  by  Regulation ;

2 )    standardisation  inspections ;                                                       or

3 )    inspections  of  an  organisation  to  verify  compliance  with  the  applicable  requirements  in  potentially  unsafe  situations.

ARO. RAMP. 105   Prioritisation  Criteria

a )   The  GDCA  shall  provide  inspection  with  a  List  of  Operators  or  aircraft  identified  as  presenting  a  potential  risk,  for  the  prioritisation  of  Ramp  Inspections ;

b )   This  list  shall  include :

1 )    Operators  of  aircraft  identified  on  the  basis  of  the  analysis  of  available  data  in  accordance  with  ARO. RAMP. 150 ( b )( 4 ) ;

2 )    Operators  or  aircraft  communicated  to  the  GDCA  and  identified  on  the  basis  of :

- An  opinion  expressed  within  the  context  of  the  implementation of  CR - EC No 2111 / 2005  that  further  verification  of  effective  compliance  with  relevant  safety  standards  through  systematic  ramp  inspections  is  necessary.

3 )    aircraft  operated  into  the  territory  subject  to  the  provisions  by  operators  included  in  Annex  B  of  the  list  of  operators  subject  to  an  operating  ban  pursuant  to  Regulation ;

4 )    aircraft  used  by  a  third - country  operator  that  operates  into,  within  or  out  of  the  territory  subject  to  the  provisions  for  the  first  time  or  whose  authorisation  issued  in  accordance  with  ART. GEN. 205  is  limited  or  reinstated  after  suspension  or  revocation.

c )   This  list  shall  be  produced,  in  accordance  with  procedures  established  by  the  EASA, after  every  update  of  the  Community  List  of  Operators  subject  to  an  operating  ban  pursuant  to                  CR - EC  No  2111 / 2005,  and  in  any  case  at  least  once  every  4  ( four )  months.

ARO. RAMP. 110   Collection  of  Information

The  GDCA  shall  collect  and  process  any  information  deemed  useful  for  conducting  Ramp  Inspections.

ARO. RAMP. 115   Qualification  of  Ramp  Inspectors

a )   The  GDCA  with  cooperation  the  EASA  shall  have  qualified  inspectors  to  conduct  Ramp  Inspections ;

b )   Ramp  inspectors  shall :

1 )    possess  the  necessary  aeronautical  education  or  practical  knowledge  relevant  to  their  area(s)  of  inspection ;

2 )    have  successfully  completed :

( i )    appropriate  specific  theoretical  and  practical  training,  in  one  or  more  of  the  following  areas  of  inspection :

- Flight  Deck ;

- Cabin  Safety ;

- Aircraft  Condition ;

- Cargo.

( ii )   appropriate  on - the - job  training  delivered  by  a  Senior  Ramp  Inspector  appointed  by  the  GDCA  of  RA ;

3 )    maintain  the  validity  of  their  qualification  by  undergoing  recurrent  training  and  by  performing  a  minimum  of  12  inspections  in  every  12-month  period.

c )   The  training  in  ( b )( 2 )( i )  shall  be  delivered  by  the  GDCA  of  RA,  or  by  any  training  organisation  approved  in  accordance  with  ARO. RAMP. 120 ( a ) ;

d )   The  EASA  shall  develop  and  maintain  training  syllabi  and  promote  the  organisation  of  training  courses  and  workshops  for  inspectors  to  improve  the  understanding  and  uniform  implementation  of  this  Subpart ;

e )   The  EASA  shall  facilitate  and  coordinate  an  inspector  exchange  programme  aimed  at allowing  inspectors  to  obtain  practical  experience  and  contributing  to  the  harmonisation  of  procedures.

ARO. RAMP. 120   Approval  of  Training  Organisations

a )   The  GDCA  shall  approve  a  Training  Organisation,  having  its  principal  place  of  business  in  the  territory  of  Republic  of  Armenia  or  the  respective  EASA  Member State,  when  satisfied  that  the  training  organisation :

1 )    has  nominated  a  head  of  training  possessing  sound  managerial  capability  to ensure  that  the  training  provided  is  in  compliance  with  the  applicable  requirements ;

2 )    has  available  training  facilities  and  instructional  equipment  suitable  for  the  type  of  training  provided ;

3 )    provides  training  in  accordance  with  the  syllabi  developed  by  the  EASA  in  accordance  with  ARO. RAMP. 115 ( d ) ;

4 )    uses  qualified  training  instructors.

b )   If  so  requested  by  the  GDCA,  the  verification  of  compliance  and  continuous  compliance  with  the  requirements  referred  to  in  ( a )  shall  be  performed  by  the  GDCA ;

c )   The  training  organisation  shall  be  approved  to  provide  one  or  more  of  the  following types  of  training :

- initial  theoretical  training ;

- initial  practical  training ;

- recurrent  training.

ARO. RAMP. 125   Conduct  of  Ramp  Inspections

a )   Ramp  Inspections  shall  be  performed  in  a  standardised  manner  using  the  form  established  in  either  Appendix  III  ( Part - ORO )  or  Appendix  IV  ( Part - CAT ) ;

b )   When  performing  a  Ramp  Inspection,  the  inspector(s)  shall  make  all  possible  efforts  to  avoid  an  unreasonable  delay  of  the  aircraft  inspected ;

c )   On  completion  of  the  Ramp  Inspection,  the  Pilot - in - Command  or,  in  his / her  absence, another  member  of  the  flight  crew  or  a  representative  of  the  Operator  shall  be  informed  of  the  Ramp  Inspection’s  results  using  the  form  established  in  Appendix  III  ( Part - ORO ).

ARO. RAMP. 130   Categorization  of  Findings

For  each  inspection  item,  3  ( three )  categories  of  possible  non-compliance  with  the  applicable  requirements  are  defined  as  findings.

Such  findings  shall  be  categorised  as  follows :

1 )    a  Category  3  Finding  is  any  detected  significant  non-compliance  with  the  applicable requirements  or  the  terms  of  a  certificate  that  has  a  major  influence  on  safety ;

2 )    a  Category  2  Finding  is  any  detected  non-compliance  with  the  applicable  requirements  or  the  terms  of  a  certificate  that  has  a  significant  influence  on  safety ;

3 )    a  Category  1  Finding  is  any  detected  non-compliance  with  the  applicable  requirements  or  the  terms  a  certificate  that  has  a  minor  influence  on  safety.

ARO. RAMP. 135   Follow-up  Actions  on  Findings

For  a  Category  2  or  3  Finding  the  GDCA,  or  where  relevant  the  cooperate  with  the  EASA,  shall :

1 )    communicate  the  finding  in  writing  to  the  Operator,  including  a  request  for  evidence  of  corrective  actions  taken ;                                                     and

2 )    inform  the  competent  authority  of  the  State  of  the  Operator  and,  where  relevant,  the  State  in  which  the  aircraft  is  registered  and  where  the  licence  of  the  flight crew  was  issued.  Where  appropriate,  the  GDCA  shall  request  confirmation  of  their  acceptance  of  the  corrective  actions  taken  by  the  Operator  in  accordance  with  ARO. GEN. 350   or  ARO. GEN. 355.

b )   In  addition  to  ( a ),  in  the  case  of  a  Category  3  Finding,  the  GDCA  of  RA  shall take  immediate  steps  by :

1 )    imposing  a  restriction  on  the  aircraft  flight  operation ;

2 )    requesting  immediate  corrective  actions ;

3 )    grounding  the  aircraft  in  accordance  with  ARO. RAMP. 140 ;                         or

4 )    send  this  Ramp  Inspection  Checklist  with  Category  3  Finding  to  EASA  for  imposing  an  immediate  operating  ban  in  accordance  with  Article  6  of  CR - EC No  2111 / 2005.

c )   When  the  EASA  has  raised  a  Category  3  Finding ,  it  shall  request  the  competent  authority  where  the  aircraft  is  landed  to  take  the  appropriate  measures  in  accordance  with ( b ).

ARO. RAMP. 140   Grounding  of  Aircraft

a )   In  the  case  of  a  Category  3  Finding  where  it  appears  that  the  aircraft  is  intended  or  is  likely  to  be  flown  without  completion  by  the  Operator  or  owner  of  the  appropriate  corrective  action,  the  GDCA  shall :

1 )    notify  the  Pilot - in - Command / Commander  or  the  Operator  that  the  aircraft  is  not permitted  to  commence  the  flight  until  further  notice ;              and

2 )    ground  that  aircraft.

b )   The  competent  authority  of  the  State  where  the  aircraft  is  grounded  shall  immediately  inform  the  competent  authority  of  the  State  of  the  Operator  and  of  the  State  in  which  the  aircraft  is  registered,  if  relevant,  and  the  EASA  in  the  case  of  a grounded  aircraft  used  by  a  third - country  Operator ;

c )   The  GDCA  shall,  in  coordination  with  the  State  of  the  Operator  or  the  State  of  Registry,  prescribe  the  necessary  conditions  under  which  the  aircraft  can  be  allowed  to  take-off ;

d )   If  the  non-compliance  affects  the  validity  of  the  certificate  of  airworthiness  of  the  aircraft,  the  grounding  shall  only  be  lifted  by  the  GDCA  when  the  operator  shows  evidence  that :

1 )    compliance  with  the  applicable  requirements  has  been  re-established ;

2 )    it  has  obtained  a  permit - to - fly  in  accordance  with  CR - EC No  1702 / 2003  ( 5 ), for  aircraft  registered  in  a  EASA  Member  State ;

3 )    a  permit - to - fly  or  equivalent  document  of  the  State  of  Registry  or  the  State  of  the  Operator  for  aircraft  registered  in  a  third  country  and  operated  by  an  EU  or  a  third  country  operator ;                                                                 and

4 )    permission  from  third  countries  which  will  be  overflown,  if  applicable.

ARO. RAMP. 145   Reporting

a )   Information  collected  in  accordance  with  ARO. RAMP. 125 ( a )  shall  be  entered  into  the GDCA of  RA  centralised  database ( which  is  directly  connected  to  EASA  Centralised  Database )  referred  to  in  ARO. RAMP. 150 ( b ) ( 2 ),  within  21  calendar  days  after  the  inspection ;

b )   The  GDCA  shall  enter  into  the  centralised  database  any  information  useful  for  the  application  of  Air  Operations  Regulation   and  its  Implementing  Rules  and  for  the  accomplishment  by  the  EASA  of  the  tasks  assigned  to  it  by  this  Annex  II,  including  the  relevant  information  referred  to  in  ARO. RAMP. 110 ;

c )   Whenever  the  information  as  referred  to  in  ARO. RAMP. 110  shows  the  existence  of  a potential  safety  threat,  such  information  shall  also  be  communicated  to  GDCA  and  the  EASA  without  delay ;

d )   Whenever  information  concerning  aircraft  deficiencies  is  given  by  a  person  to  the  GDCA,  the  information  referred  to  in  ARO. RAMP. 110  and   ARO. RAMP. 125 ( a )  shall  be  de -identified  regarding  the  source  of  such  information.

ARO. RAMP. 150   Agency  ( EASA )  Coordination  Tasks

a )   The  EASA  shall  manage  and  operate  the  tools  and  procedures  necessary  for  the  storage  and  exchange  of :

1 )    the  information  referred  to  in  ARO. RAMP. 145,  using  the  forms  as  established  in  Appendices  III  and  IV ;

2 )    the  information  provided  by  third  countries  or  international  organisations  with  whom  appropriate  agreements  have  been  concluded  with  the  GDCA  of  RA.

b )   This  management  shall  include  the  following  tasks :

1 )    store  data  from  the  EASA  Member  States  relevant  to  the  safety  information  on  aircraft landing  at  aerodromes  located  in  the  territory  subject  to  the  provisions  of  the  Treaty ;

2 )    develop,  maintain  and  continuously  update  a  centralised  database  containing  all  the  information  referred  to  in  ( a )( 1 )  and  ( 2 ) ;

3 )    provide  necessary  changes  and  enhancements  to  the  database  application ;

4 )    analyse  the  centralised  database  and  other  relevant  information  concerning  the  safety  of  aircraft  and  of  Air  Operators  and,  on  that  basis :

( i )    advise  the  Commission  and  the  competent  authorities  on  immediate  actions  or  follow-up  policy ;

( ii )   report  potential  safety  problems  to  the  EASA  and  to  the  competent  authorities ;

( iii )  propose  coordinated  actions  to  the  EASA  and  to  the  competent  authorities,  when  necessary  on  safety  grounds,  and  ensure  coordination  at  the  technical  level  of  such  actions ;

5 )    liaise  with  EASA,  international  organisations  and  third  country  competent  authorities  on  information  exchange.

ARO. RAMP. 155   Annual   Report

The  GDCA  of  RA  shall  prepare  and  submit  to  the  EASA  an  Annual  Report  on  the  Ramp Inspection  system  containing  at  least  the  following  information :

a )   status  of  the  progress  of  the  system ;

b )   status  o f the  inspections  performed  in  the  year ;

c )   analysis  of  the  inspection  results  with  indication  of  the  categories  of  findings ;

d )   actions  taken  during  the  year ;

e )   proposals  for  further  improving  the  ramp  inspection  system ;                            and

f )   annexes  containing  lists  of  inspections  sorted  out  by  State  of  operation,  aircraft  type, operator  and  ratios  per  item.

ARO. RAMP. 160     Information  to  the  Public  and  Protection  of  Information

a )  GDCA  shall  use  the  information  received  by  them  pursuant  to  ARO. RAMP. 105  and        ARO. RAMP. 145  solely for  the  purpose of  CR - EC No 216 / 2008  and  its  Implementing  Rules  and  shall  protect  it  accordingly ;

b )  The EASA  shall  publish  an  aggregated  information  report  annually  that  shall  be  available  to  the  public  containing  the  analysis of  the  information  received  in  accordance  with                      ARO. RAMP. 145.  The  report  shall  be  simple  and  easy  to  understand,  and  the  source  of  the information  shall  be  de-identified.

INTENTIONALLY            LEFT            BLANK